### Commands

Router

// setup

en

conf t

no ip domain-lookup

hostname <name>

banner motd #<text>#

interface <g3>

description <text>

ip address <ip> <snm>

no shutdown

exit

<repeat>

// pw security

enable secret <setSecret>

service password-encryption

(security passwords min-length <no>)

// domain

ip domain-name <doma.in>

username netadmin secret <secret>

// pw login

line console 0

password <pw>

login

exit

// ssh

line vty n n <0 15>

transport input ssh

login local

exit

// generate rsa key pair

crypto key generate rsa

<nr> // 1024

// save

exit

copy running-config startup-config

Switch

# setup

// vlan interface

interface vlan <no>

# descr

# ip

#

#

ip default-gateway <ipdg>

# pw security

# domain

#pw login

# ssh

# rsa

int range <range1>, <range2>, <range3>

description Unused switch ports

shutdown

// save with “end”

## Instructions

In this assessment you will configure the R1 router and S1 switch, as you have done in the activities in this course. You will also connect two PCs using a switch and a router that are in the main wiring closet. You will subnet the 172.16.1.0/24 network to provide IPv4 addresses for two subnets that will support the required number of hosts. The larger subnet (LAN 1) requires 60 hosts and the smaller subnet (LAN 2) requires 20 hosts.

#### Part 1: Build the Network

a. Build the network according to the logical topology by placing the required equipment in the wiring closet equipment rack.
b. Cable the network devices in the closet as shown in the topology diagram.
c. Connect the hosts as shown in the topology diagram.

#### Part 2: Develop an IP Addressing Scheme

In this part of the assessment, you will develop an IP addressing scheme. You will subnet an IPv4 network to create two subnets with the required number of hosts. You will also subnet an IPv6 network. You will then assign the addresses according to the requirements below.

Work with the following information:

- IPv4 Network: 172.16.1.0/24

- Required number of hosts in IPv4 LAN 1: 60

- Required number of hosts in IPv4 LAN 2: 20

a. Record your subnet assignments according to the following requirements.

1) Assign the first IPv4 address of each subnet to a router interface

- LAN 1 is hosted on R1 G0/0/1

- LAN 2 is hosted on R1 G0/0/0

2) Assign the last IPv4 address of each subnet to the PC NIC.
3) Assign the second IPv4 address of LAN 1 to S1 SVI.

#### Part 3: Configure Basic Device Settings

Network devices must be configured over a direct console connection. Connectivity between the hosts should be established.

##### Step 1: Configure Basic Settings

a. Disable DNS lookup on R1 and S1
b. Configure router hostname using the name R1.
c. Configure switch hostname using the name S1.
d. Configure an appropriate banner on R1 and S1.
e. Allow console logins with the password C@nsPassw!

##### Step 2: Configure Interfaces

a. Configure R1 G0/0/0 and G0/0/1 interfaces using the addressing from the previous part of this assessment:

- Interface description

- IPv4 address / subnet mask

b. Configure the S1 VLAN 1 SVI interface using the addressing from the previous part of this assessment:

- Interface description

- IPv4 address / subnet mask

- The switch should be reachable from devices on other networks.

#### Part 4: Configure Security Settings on R1 and S1

##### Step 1: Configure enhanced password security

a. Configure DontTellAnyone as the encrypted privileged EXEC password
b. Encrypt all plaintext passwords
c. Set minimum password length to 10 on R1.

##### Step 2: Configure SSH on R1 and S1

a. Configure netsec.com as the domain name
b. Configure a local user netadmin with the secret password Ci$co12345
c. Set login on vty lines to use local database.
d. Configure the vty lines to accept SSH access only.
e. Generate an RSA crypto key using 1024 bits modulus.

##### Step 3: Secure switch ports on S1

a. Shut down all unused ports on S1.
b. Enter descriptions for all unused switch ports to indicate that they are intentionally shutdown.

#### Part 5: Configure the Hosts and Verify Connectivity

Configure both hosts with the IPv4 addresses that were assigned in Part 2 of this assessment.

#### Part 1: Build the Network

Placing Switch S1 and Router R1 to wiring closet equipment rack.

Using Copper Straight-Through cable to connect PC-A (FastEthernet0 port) and S1 (FastEthernet0/6 port)

Same as above, using Copper Straight-Through cable to connect all devices as shown in the topology diagram.

ITN Final PT Skills Assessment (PTSA)

Turn-on PCs and Router R1

#### Part 2: Develop an IP Addressing Scheme

| Item | Requirements | IPv4 Address |
| --- | --- | --- |
| Network Address | 172.16.1.0/24 |  |
| LAN 1 subnet host requirements | 60 | 172.16.1.0/26 SM: 255.255.255.192 |
| LAN 2 subnet host requirements | 20 | 172.16.1.64/27 SM: 255.255.255.224 |
| R1 G0/0/1 | First host address in LAN 1 subnet | 172.16.1.1 |
| R1 G0/0/0 | First host address in LAN 2 subnet | 172.16.1.65 |
| S1 SVI | Second host address in LAN 1 subnet | 172.16.1.2 |
| PC-A | Last host address in LAN 1 subnet | 172.16.1.62 |
| PC-B | Last host address in LAN 2 subnet | 172.16.1.94 |

Using line console to connect PC-A and Router

Click to PC-A --> Terminal app --> click OK

Router R1 configuration script

enable

configure terminal

no ip domain-lookup

hostname R1

banner motd #Unauthorized access to this device is prohibited!#

interface g0/0/0

description Connect to Subnet B

ip address 172.16.1.65 255.255.255.224

no shutdown

exit

interface g0/0/1

description Connect to Subnet A

ip address 172.16.1.1 255.255.255.192

no shutdown

exit

enable secret DontTellAnyone

service password-encryption

security passwords min-length 10

ip domain-name netsec.com

username netadmin secret Ci$co12345

line console 0

password C@nsPassw!

login

exit

line vty 0 15

transport input ssh

login local

exit

crypto key generate rsa

1024

exit

copy running-config startup-config

#### Configuration for S1

Then, using Console cable to connect PC-B and Switch
To show Console port on Switch, Right click Switch --> Inspect Rear --> Console port

Switch S1 configuration script

enable

configure terminal

no ip domain-lookup

hostname S1

banner motd #Unauthorized access to this device is prohibited!#

interface vlan 1

description Switch Subnet A

ip address 172.16.1.2 255.255.255.192

no shutdown

exit

ip default-gateway 172.16.1.1

enable secret DontTellAnyone

service password-encryption

ip domain-name netsec.com

username netadmin secret Ci$co12345

line console 0

password C@nsPassw!

login

exit

line vty 0 15

transport input ssh

login local

exit

crypto key generate rsa

1024

int range f0/1 - 4, f0/7 - 24, g0/1 - 2

description Unused switch ports

shutdown

end

copy running-config startup-config

#### Part 5: Configure the Hosts and Verify Connectivity

On PCs, go to Desktop tab --> IP Configuration menu

| PC-A Network Configuration |  |
| --- | --- |
| IPv4 Address | 172.16.1.62 |
| Subnet Mask | 255.255.255.192 |
| IPv4 Default Gateway | 172.16.1.1 |

| PC-B Network Configuration |  |
| --- | --- |
| IPv4 Address | 172.16.1.94 |
| Subnet Mask | 255.255.255.224 |
| IPv4 Default Gateway | 172.16.1.65 |